# DUAL•LENS technology: leading-edge lighting technique

DUAL•LENS represents an innovative lens technology,  specifically developed for the lighting of large spaces. The linear lens combines superior comfort with directional light distribution for the most diverse applications. ETAP applies DUAL•LENS technology in three luminaire series: E2, E4 and E5.

One lens, two functions

The unique surface structure of the linear lens has a double function. The structure on the outside of the lens reduces the luminance of the LEDs (UGR < 22 or < 25), without compromising the efficiency.  The structure on the inside determines the light distribution. Depending on the kind of space you'd like to light, you can choose from four types: wide angle, narrow angle, asymmetric and double asymmetric optics.

High efficiency, long service life

DUAL•LENS technology combines a unique lens structure with medium power LEDs, which stand out because of their high efficiency, their low consumption and their long service life. The ceramic LEDs are very stable, also with high temperatures: after 50,000 hours they keep up to 98% of their luminous intensity (LLMF - Lamp Lumen Maintenance Factor).

Three series of luminaires

ETAP applies DUAL•LENS technology in three luminaire series.

-The E4 series offers an efficient solution for the lighting of production halls and warehouses, workstations and store shelves. Depending of the application you can choose between individual luminaires and continuous light lines (up to 6000 lm/m). The housing is white-painted extruded aluminium.

- The E5 series comprises a steel base unit on which separate white-painted extruded aluminium LED modules can be mounted. The variable spacing (multiples of 50 cm) guarantee optimum flexibility. E5 achieves luminous fluxes up to 6000 lm/m and is suitable for industrial environments such as production or storage areas. In renovation projects the LED modules can also be mounted on existing E3 and E5 profiles.

- For damp and/or highly soiled spaces, ETAP developed the E2-series: an LED lighting solution for the chemical, pharmaceutical and food industries, among others. The robust, aluminium housing is resistant against numerous aggressive chemicals. Additionally the LEDs, which are sensitive to corrosion among others, are safely enclosed in a sealed compartment (compliant with corrosion test EN60038-2-60). The result? A series of impact-resistant (IK07/IK08), dust-proof and watertight (IP66) luminaires that furthermore operate trouble-free in a wide temperature range.

Do not miss ETAP's innovations at light+building. Visit us in Hall 4.1, stand D70.

Or book an appointment by sending an e-mail to press@etaplighting.com